# Installing and Testing Verilator on Windows Using WSL2

## Overview

This guide walks through:

- Installing Windows Subsystem for Linux (WSL2)

- Installing Ubuntu

- Installing Verilator

- Creating a simple Verilog design

- Simulating the design with Verilator

These instructions have been tested on Windows 10 and Windows 11.

# Part 1: Verify Hardware Virtualization

Before installing WSL2, verify that CPU virtualization is enabled.

- Press Ctrl + Shift + Esc to open Task Manager.

- Select the Performance tab.

- Select CPU.

- Verify:

Virtualization: Enabled

If virtualization is disabled, enable Intel VT-x or AMD-V in your computer’s BIOS/UEFI settings before continuing.

# Part 2: Enable Required Windows Features

Open PowerShell as Administrator:

- Press the Windows key.

- Type:

PowerShell

- Right-click Windows PowerShell.

- Select Run as administrator.

Enable WSL:

Enable-WindowsOptionalFeature -Online -FeatureName Microsoft-Windows-Subsystem-Linux -All

Enable the Virtual Machine Platform:

Enable-WindowsOptionalFeature -Online -FeatureName VirtualMachinePlatform -All

Restart your computer when prompted.

# Part 3: Install and Configure WSL2

After rebooting, open PowerShell and run:

wsl --update

Set WSL2 as the default version:

wsl --set-default-version 2

Verify the installation:

wsl --status

Expected output should indicate:

Default Version: 2

# Part 4: Install Ubuntu

Install Ubuntu:

wsl --install -d Ubuntu

Alternatively, Ubuntu can be installed from the Microsoft Store.

After installation completes, launch Ubuntu from the Start Menu.

# Part 5: Complete Ubuntu First-Time Setup

The first launch may take several minutes.

Eventually you should see:

Please create a default UNIX user account.
Enter new UNIX username:

Choose:

- A username

- A password

Remember this password. You will use it whenever you run commands with sudo.

## Troubleshooting Ubuntu Startup

### Ubuntu hangs during first launch

If Ubuntu appears stuck:

- Close Ubuntu.

- Open PowerShell.

- Run:

wsl --shutdown

- Launch Ubuntu again.

The setup process should continue and eventually prompt for a username and password.

### Message: “waiting for OOBE command to complete”

This indicates Ubuntu’s first-time setup has not yet finished.

Run:

wsl --shutdown

Then relaunch Ubuntu and allow several minutes for setup to complete.

# Part 6: Install Verilator

Once Ubuntu is running and you see a Linux prompt similar to:

username@DESKTOP-XXXX:~$

Update package information:

sudo apt update

Install Verilator and supporting tools:

sudo apt install -y verilator make g++ git gtkwave

Verify the installation:

verilator --version

Example output:

Verilator 5.032

# Part 7: Create a Test Project

Create a project directory:

mkdir ~/verilator_test
cd ~/verilator_test

# Part 8: Create a Verilog Design

Create a file:

nano counter.sv

Paste the following code:

module counter (
    input logic clk,
    input logic rst,
    output logic [3:0] count
);

always_ff @(posedge clk) begin
    if (rst)
        count <= 0;
    else
        count <= count + 1;
end

endmodule

Save the file:

- Ctrl+O

- Enter

- Ctrl+X

Verify the file exists:

ls

Expected:

counter.sv

# Part 9: Create a C++ Testbench

Create the testbench:

nano sim_main.cpp

Paste:

#include "Vcounter.h"
#include "verilated.h"
#include <iostream>

int main(int argc, char** argv) {
    Verilated::commandArgs(argc, argv);

    Vcounter* top = new Vcounter;

    top->clk = 0;
    top->rst = 1;

    for (int i = 0; i < 20; i++) {
        top->clk = !top->clk;

        if (i == 2)
            top->rst = 0;

        top->eval();

        if (top->clk) {
            std::cout << "count = "
                      << (int)top->count
                      << std::endl;
        }
    }

    delete top;
    return 0;
}

Save the file:

- Ctrl+O

- Enter

- Ctrl+X

Verify both files exist:

ls

Expected:

counter.sv
sim_main.cpp

# Part 10: Build the Simulation

Compile the design:

verilator --cc --exe --build counter.sv sim_main.cpp

A successful build creates an obj_dir directory.

# Part 11: Run the Simulation

Execute:

./obj_dir/Vcounter

Expected output:

count = 0
count = 1
count = 2
count = 3
count = 4
count = 5
count = 6
count = 7
count = 8

# Installation Verification Checklist

A successful installation should satisfy all of the following:

verilator --version

Returns:

Verilator 5.x

g++ --version

Returns a GCC version.

gtkwave --version

Returns a GTKWave version.

./obj_dir/Vcounter

Runs successfully and displays an incrementing count.

If all four checks pass, the Verilator installation is ready for coursework.